| Section | Item | What to Check / Evidence | Risk if Non-Compliant | Status | Priority | Owner | Corrective Action & Due Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| External Grounds | Perimeter litter | Absence of food debris & packaging | Rodent & fly attraction |  |  |  |  |
| External Grounds | Landscaping management | Low shrubs, trimmed edges, no dense groundcover | Rodent harborage |  |  |  |  |
| External Grounds | Waste compactor / bins | Lids closed, area clean, concrete intact | Rodent/cockroach breeding |  |  |  |  |
| External Grounds | Delivery bays | Swept daily, no spill residues | Rodent ingress / flies |  |  |  |  |
| External Structure | Dock seals & doors | No gaps when closed; seals pliable | Rodent/insect entry |  |  |  |  |
| External Structure | Lighting management | Correct wavelength; avoid attracting flying insects (shields) | Increased night pest pressure |  |  |  |  |
| External Structure | Bird activity & nesting | No active nests; ledges treated | Fouling, disease |  |  |  |  |
| External Structure | Cracks / wall penetrations | Sealed with durable materials | Rodent/cockroach ingress |  |  |  |  |
| Building Interior – Reception/Office | Food consumption policy | Defined & enforced (food only in kitchen areas) | Rodent/roach dispersal |  |  |  |  |
| Building Interior – Office | Desk hygiene | No open food containers overnight | Attraction of pests |  |  |  |  |
| Warehouse / Stock Areas | Product rotation | FIFO visible; expired goods flagged | Stored product pest risk |  |  |  |  |
| Warehouse / Stock Areas | Pallet stacking | 150–450 mm off walls; clear inspection aisles | Hidden rodent runway |  |  |  |  |
| Warehouse / Stock Areas | Ceiling void access | Locked, controlled access | Undetected rodent harborage |  |  |  |  |
| Warehouse / Stock Areas | Spill response process | Documented & executed quickly | Residual attractants |  |  |  |  |
| Warehouse / Stock Areas | Incoming goods inspection | Random checks for pest signs; documented | Infestation introduction |  |  |  |  |
| Facility Services | Break room sanitation | Surfaces clean; refrigeration defrosted | Roach/ant attraction |  |  |  |  |
| Facility Services | Vending machines | Base clean; no debris underneath | German roach harborage |  |  |  |  |
| Facility Services | Restroom hygiene | Drains clear, no fruit/drain flies | Reputational impact |  |  |  |  |
| Structural Integrity | Ceiling tiles | Intact; stained tiles investigated (leaks) | Moisture pest conducive |  |  |  |  |
| Structural Integrity | Pipe chases / risers | Sealed; no debris | Rodent highways |  |  |  |  |
| Proofing | Air curtains (if used) | Functional coverage at loading doors | Flying insect ingress |  |  |  |  |
| Proofing | Rodent burrows outdoors | None; treated & collapsed if present | Colony establishment |  |  |  |  |
| Monitoring Devices | Rodent bait stations map | Up-to-date site map; devices numbered | Lost trend continuity |  |  |  |  |
| Monitoring Devices | Bait / block condition | Fresh, unlabeled tampering absent | Reduced control efficacy |  |  |  |  |
| Monitoring Devices | Snap traps / multi-catch | Clean, functioning, recorded | Missed captures |  |  |  |  |
| Monitoring Devices | Insect light traps (ILTs) | Correct height, away from open doors, catch tray logged monthly | Flying insect proliferation |  |  |  |  |
| Monitoring Devices | Sticky monitors | Dated, replaced per rotation schedule | Loss of trend data |  |  |  |  |
| Sanitation | Cleaning scope & schedule | Clear matrix (daily/weekly/monthly); audited | Residual food sources |  |  |  |  |
| Sanitation | High dust accumulation | Over racks / beams minimized | Stored product pests |  |  |  |  |
| High-Risk Pests | Rodent indicators | Fresh droppings, gnaw, smear marks absence | Active infestation risk |  |  |  |  |
| High-Risk Pests | Cockroach indicators | Live/dead, oothecae, fecal spotting | Rapid growth cycle |  |  |  |  |
| High-Risk Pests | Birds | Feathers, droppings, nesting attempt | Contamination hazard |  |  |  |  |
| High-Risk Pests | Stored product pests | Webbing, larvae in packaging seams | Product loss |  |  |  |  |
| Chemical Control | Pesticide storage (technician) | Locked, labelled, SDS accessible | Regulatory breach |  |  |  |  |
| Documentation | Service reports | Present, signed, action items tracked | Gaps unresolved |  |  |  |  |
| Documentation | Trend analysis (quarterly) | Charts for rodents / insects maintained | Blind to escalation |  |  |  |  |
| Documentation | Corrective action closure | Dated & closed in log | Recurring issues |  |  |  |  |
| Training & Awareness | Staff pest reporting process | Communicated & understood | Delayed detection |  |  |  |  |
| Training & Awareness | New hire induction | Pest prevention basic module | Persistent bad habits |  |  |  |  |
| Safety & Compliance | Signage on devices | “Do not move” / hazard warnings visible | Tampering & liability |  |  |  |  |
| Review & Follow-Up | Priority action summary | Top 5 with target dates | Drift in compliance |  |  |  |  |
| Review & Follow-Up | Next audit date | Scheduled (quarterly recommended) | Lost program continuity |  |  |  |  |