| Clause / Area | Item | What to Check / Evidence | Risk / Standard Impact | Status | Priority |  | Owner | Corrective Action & Due Date |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Governance & Program | Written pest management program | Scope, frequency, target pests, escalation path documented | Non-conformance (program absence) |  |  |  |  |  |
| Governance & Program | Service provider competence | Technician training records, certifications | Ineffective controls |  |  |  |  |  |
| Governance & Program | Contract / SLA | Defined response times, reporting expectations | Delayed remediation |  |  |  |  |  |
| Governance & Program | Risk assessment (zoning) | High / medium / low risk zones defined (e.g. open product areas) | Misallocated resources |  |  |  |  |  |
| Governance & Program | Device location rationale | Map includes risk-based spacing & unique IDs | Inefficient coverage |  |  |  |  |  |
| Governance & Program | Document control | Versioned procedures, review cycle set | Outdated methods |  |  |  |  |  |
| Building Exterior | Perimeter hygiene | No spilled ingredients / pallet debris | Rodent & bird pressure |  |  |  |  |  |
| Building Exterior | Proofing integrity | Doors self-closing; seals & brushes intact | Rodent/ insect ingress |  |  |  |  |  |
| Building Exterior | Bird deterrents | Netting/spikes/line intact; no nesting | Product contamination |  |  |  |  |  |
| Building Exterior | Roof & gutter condition | Clean, no pooling water | Mosquito / structural pests |  |  |  |  |  |
| Raw Material Intake | Trailer / container inspection | Random pest check log maintained | Infested goods entry |  |  |  |  |  |
| Raw Material Intake | Pallet & packaging control | Non-food-grade waste segregated quickly | Hidden pest harbor |  |  |  |  |  |
| Internal Structure | Floor-to-wall junctions | Sealed, cleanable coving | Harborage & sanitation difficulty |  |  |  |  |  |
| Internal Structure | Overhead pipes & cables | No product dust accumulation | Stored product pest attractant |  |  |  |  |  |
| Internal Structure | Wall penetrations | Properly sealed (no gaps) | Rodent runways |  |  |  |  |  |
| Internal Structure | Drains | Clean, traps functioning, no larvae | Drain fly contamination |  |  |  |  |  |
| Environmental Controls | Air handling | Positive pressure in high-hygiene zones | Airborne insect ingress |  |  |  |  |  |
| Environmental Controls | Strip curtains / air curtains | Intact, clean, properly mounted | Flying insect ingress |  |  |  |  |  |
| Environmental Controls | Temperature / humidity logs | Within spec; anomalies investigated | Pest conducive climate |  |  |  |  |  |
| Proofing & Exclusion | Door discipline | Doors not propped open; audit observed | Pest entry |  |  |  |  |  |
| Proofing & Exclusion | Sieve / mesh integrity | Mesh size appropriate (≤2 mm in sensitive areas) | Small insect ingress |  |  |  |  |  |
| Monitoring – Rodents | External bait stations | Intact, anchored, locked, numbered | Tampering, missed activity |  |  |  |  |  |
| Monitoring – Rodents | Internal detection (non-toxic) | Correct placement near runways; recorded | Hidden activity trend loss |  |  |  |  |  |
| Monitoring – Rodents | Bait freshness & usage records | Change-outs dated; consumption logged | Overlooked escalation |  |  |  |  |  |
| Monitoring – Insects | ILT placement | Not visible from exterior; not over exposed product | Contamination risk |  |  |  |  |  |
| Monitoring – Insects | ILT catch analysis | Species recorded monthly (trend graph) | Blind to seasonal spike |  |  |  |  |  |
| Monitoring – Insects | Pheromone traps (SPP) | Data cards dated; lure change schedule adhered | Missed stored pest onset |  |  |  |  |  |
| Monitoring – Insects | Floor / pit traps (if used) | Cleaned, contents logged | Missed crawling insect trends |  |  |  |  |  |
| Monitoring – Birds | Indoor bird ingress log | Incidents recorded & closed | Persistent contamination risk |  |  |  |  |  |
| High-Risk Zones | Open product areas | Free from live/dead insects; no rodent indicators | Direct contamination |  |  |  |  |  |
| High-Risk Zones | Rework / waste holding | Containers sealed & cleared promptly | Fly / roach attractant |  |  |  |  |  |
| High-Risk Zones | Allergen storage / segregation | Pest activity absent; sealed | Allergen recall risk |  |  |  |  |  |
| Sanitation Integration | Cleaning schedule alignment | Pest hotspots included in master sanitation schedule | Harbor persistence |  |  |  |  |  |
| Sanitation Integration | Spill management response | Documented ≤15 min response target | Insect/rodent attraction |  |  |  |  |  |
| Sanitation Integration | Waste compactor hygiene | Clean pad & drainage; schedule documented | Rodent breeding site |  |  |  |  |  |
| Chemical Controls | Pesticide list & SDS | Approved list current; SDS accessible | Regulatory breach |  |  |  |  |  |
| Chemical Controls | Application records | Target pest, actives, lot numbers, technician signature | Traceability gap |  |  |  |  |  |
| Chemical Controls | Non-target risk mitigation | No routine broad-spray in open product zones | Contamination & non-compliance |  |  |  |  |  |
| Allergen / Food Safety Interface | Device placement vs product safety | No bait stations above exposed product; only detection devices internally | Product contamination |  |  |  |  |  |
| Documentation & Trending | Site map (revision) | Date-stamped; all devices present & accurate | Audit non-conformance |  |  |  |  |  |
| Documentation & Trending | Activity trend charts | Rodent/insect trends graphed (≥12 months) | Escalation unnoticed |  |  |  |  |  |
| Documentation & Trending | KPI tracking | Response time, closure time, infestation severity index | Lack of performance oversight |  |  |  |  |  |
| Documentation & Trending | Root cause analyses | Completed for repeat activity & product risk events | Recurring incidents |  |  |  |  |  |
| Documentation & Trending | Corrective & preventive actions (CAPA) | CAPA logged with verification step | Incomplete resolution |  |  |  |  |  |
| Training & Competence | Staff awareness training | Annual pest awareness & reporting protocol | Late reporting |  |  |  |  |  |
| Training & Competence | Internal audits | At defined frequency (e.g. quarterly) documented | Program drift |  |  |  |  |  |
| Training & Competence | Contractor controls | Visitor / contractor policy prevents entry with infested materials | Pest introduction |  |  |  |  |  |
| Incident Management | Contamination protocol | Defined isolation / hold procedure for pest-related contamination | Unsafe release |  |  |  |  |  |
| Incident Management | Escalation ladder | Clear chain (Operator → QA → ASC Technician) | Delay in mitigation |  |  |  |  |  |
| Continuous Improvement | Annual program review | Joint review: hotspots, device optimization | Static ineffective layout |  |  |  |  |  |
| Continuous Improvement | Technology enhancements | Consider remote monitoring / digital logs | Manual data lag |  |  |  |  |  |
| Audit Readiness | Last 3 service reports | Accessible, signatures present | Incomplete evidence |  |  |  |  |  |
| Audit Readiness | Last external audit findings | Prior pest findings closed | Repeat non-conformance |  |  |  |  |  |
| Final Summary | High severity open items | List with deadlines & owners | Regulatory / certification risk |  |  |  |  |  |
| Final Summary | Next review date | Scheduled (monthly internal / quarterly strategic) | Lapsed oversight |  |  |  |  |  |