| Section | Item | Audit Point / Evidence to Verify | Risk (Q/FS/Reg/HS/Ops) | Status | Priority | Action Owner | Corrective Action / Notes | Due Date |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-Harvest Planning | Crop field pest pressure review | Field scouting records inform storage protection plan | Q/FS |  |  |  |  |  |
| Pre-Harvest Planning | Grain moisture targets | Moisture acceptance ≤ recommended spec (e.g. maize ≤13–14%) | Q/FS/Ops |  |  |  |  |  |
| Empty Bin Prep | Bin emptied & old grain removed | No residual caked grain / dust on ledges | Q/FS |  |  |  |  |  |
| Empty Bin Prep | Mechanical cleaning completed | Swept floors, brushed walls, sweep augers used | Q/FS/Ops |  |  |  |  |  |
| Empty Bin Prep | Overhead structures cleaned | Beams, catwalks, spouting free of fines/dust | Q/FS |  |  |  |  |  |
| Empty Bin Prep | Exterior pad cleanliness | Weeds, spilled grain removed around base (≥1 m clearance) | Rodent & insect harbor (FS) |  |  |  |  |  |
| Structural Integrity | Seals & gaskets | Hatches, doors, manways tight sealing | Insect ingress (FS) |  |  |  |  |  |
| Structural Integrity | Roof & sidewall leaks | No water penetration evidence / rust holes sealed | Mold / insect hotspot (FS/Q) |  |  |  |  |  |
| Structural Integrity | Aeration duct integrity | Grates clean, no fines blockage | Uneven cooling (Q/Ops) |  |  |  |  |  |
| Structural Integrity | Foundation / base | No cracks enabling moisture wicking | Moisture / mold (Q/FS) |  |  |  |  |  |
| Structural Integrity | Access ladders & safety guards | Safe for inspection; no infestation harbor | HS / Ops |  |  |  |  |  |
| Sanitation | Spillage control | Spilled grain promptly removed (inside & perimeter) | Stored product pest attraction (FS) |  |  |  |  |  |
| Sanitation | Waste disposal | Sweepings & infested material removed off-site | Reinvasion risk (FS) |  |  |  |  |  |
| Monitoring (Insects) | Probe / trap deployment | Grain probes, pitfall or pheromone traps installed per plan | Early detection (FS/Q) |  |  |  |  |  |
| Monitoring (Insects) | Inspection frequency | Recorded (e.g. weekly in warm months; monthly cool) | Undetected escalation (FS) |  |  |  |  |  |
| Monitoring (Insects) | Hot spot detection | Temperature cables / manual readings identify hotspots | Hidden infestation (FS/Q) |  |  |  |  |  |
| Monitoring (Rodents) | Perimeter bait stations | Mapped, anchored, fresh bait, non-spill | Rodent contamination (FS) |  |  |  |  |  |
| Monitoring (Rodents) | Internal activity signs | Droppings, gnawing, burrows absent | FS/Q |  |  |  |  |  |
| Monitoring (Birds) | Bird presence at roof / vents | Droppings, nesting material absent | FS / Reg |  |  |  |  |  |
| Environmental Control | Aeration plan documented | Target temperature differentials defined | Insect/metabolic heat (Q/Ops) |  |  |  |  |  |
| Environmental Control | Aeration record logs | Start/stop times, ambient & grain temp logged | Trend gap (Q/FS) |  |  |  |  |  |
| Environmental Control | Moisture trend data | Periodic moisture sampling documented | Mold/insect risk (FS/Q) |  |  |  |  |  |
| Grain Quality | Sampling method SOP | Representative core / probe procedure followed | Data reliability (Q/FS) |  |  |  |  |  |
| Grain Quality | Insect ID accuracy | Suspect insects preserved & identified (e.g. weevils vs borers) | Inappropriate treatment (FS/Q) |  |  |  |  |  |
| Grain Quality | Damaged kernels % | Monitoring above threshold triggers action | Pest proliferation (FS/Q) |  |  |  |  |  |
| Exclusion | Vent screening | Mesh intact (exclude birds & large insects) | Entry risk (FS) |  |  |  |  |  |
| Exclusion | Seal around cables/pipes | Foamed / gasketed; no gaps | Ingress point (FS) |  |  |  |  |  |
| Chemical / Fumigation | Treatment decision log | Justification (threshold, species, extent) documented | Regulatory gap (Reg/FS) |  |  |  |  |  |
| Chemical / Fumigation | Product & label compliance | Only approved actives; label directions followed | Reg / HS |  |  |  |  |  |
| Chemical / Fumigation | Phosphine management | Sealing adequacy; gas monitoring plan | HS / FS |  |  |  |  |  |
| Chemical / Fumigation | Gas monitoring records | Concentration vs time profile captured | Incomplete kill (FS) |  |  |  |  |  |
| Chemical / Fumigation | Aeration / clearance sign-off | Re-entry levels below TLV; authorized release | HS / Reg |  |  |  |  |  |
| Resistance Management | Rotation of actives | Avoid repeated single-mode fumigants | Resistance (FS) |  |  |  |  |  |
| Safety | PPE availability | Respirators (fit checked), gloves, goggles accessible | HS / Reg |  |  |  |  |  |
| Safety | Confined space protocol | Permits, attendants, gas test before entry | HS / Reg |  |  |  |  |  |
| Documentation | Master pest log | Activity counts, dates, corrective actions | Trend failure (FS/Q) |  |  |  |  |  |
| Documentation | Calibration of probes | Moisture/temp devices calibrated (dates) | Data integrity (Q/Ops) |  |  |  |  |  |
| Documentation | Training records | Staff trained on sampling & pest ID | Misclassification (FS) |  |  |  |  |  |
| Continuous Improvement | Post-season review | Losses vs causes analyzed; plan updated | Recurrence (Q/Ops) |  |  |  |  |  |
| Summary | Critical open items | Top 5 risks enumerated | Escalation delay |  |  |  |  |  |